# MRPL: Draft an approval note for PRV-1042 finding a 4% set-pressure deviation.

Generated: 2026-08-31 16:46:04

## Content

## Search knowledge base for: Draft an approval note for PRV-1042 finding a 4% set-pressure deviation.

[1] (source: sop_pressure_relief_valve.txt, score: 0.807)
SOP-14: Pressure Relief Valve Inspection and Approval Procedure Scope: This procedure applies to all pressure relief valves (PRVs) installed on process vessels and piping in the crude distillation and hydrocracking units. Inspection frequency: PRVs must be inspected every 12 months, or every 6 months for valves in hydrogen or high-H2S service. Approval note requirements: Any inspection report noting valve seat leakage, corrosion on the valve body, or a set-pressure deviation greater than 3% from nameplate rating must be escalated to the Rotating Equipment section head for review before the valve is returned to service. The approval note must include: valve tag number, last inspection date, observed deviation, inspector's recommendation, and section head sign-off. Deviations exceeding 5% require the valve to be pulled and bench-tested before any approval note can be issued. Reference: API 527, API 576.

[2] (source: sop_approval_note_format.txt, score: 0.709)
SOP-02: Standard Approval Note Format All approval notes generated from inspection findings must follow this structure: 1. Header: equipment tag, unit, date of inspection, inspector name. 2. Findings summary: 2-4 sentences describing what was observed. 3. Reference SOPs: list every SOP consulted, with section numbers. 4. Deviation assessment: state whether the finding is within tolerance, requires monitoring, or requires escalation. 5. Recommendation: one clear action - return to service, monitor, escalate, or pull for bench testing. 6. Sign-off block: preparer, reviewer, and approver fields. Approval notes are stored in the department's SOP-compliant archive and must be retrievable by equipment tag number for at least 7 years.

## Sources

Search knowledge base for: Draft an approval note for PRV-1042 finding a 4% set-pressure deviation.